VIMS Internship Handoff - FlowMeter

Main program: Code/FlowMeterMain/FlowMeterMain.ino

This document explains the final FlowMeter setup, the main wiring, the settings that can be changed in the program, how to upload the program, and how to read the logged data from the microSD card.

# 1. System Overview

The final system has three main components: an Adafruit Feather M0, a YF-DN80 flowmeter, and a DS3231 real-time clock (RTC). The LiPo battery connects to the Feather. The flowmeter is powered from the Feather BAT pin, so it receives the battery voltage directly. The microSD card is inserted into the Feather's built-in SD card slot.

The flowmeter sends a pulse signal to the Feather. A resistor voltage divider is used on the signal line before it reaches D11 so that the signal voltage is safe for the Feather's 3.3 V logic.

# 2. Wiring

| From | To | Notes |
| --- | --- | --- |
| LiPo battery | Feather battery connector | Main power for the system. |
| Feather BAT | YF-DN80 power / red wire | Supplies the battery voltage directly to the flowmeter. |
| Feather GND | YF-DN80 ground / black wire | The Feather and flowmeter must share the same ground. |
| YF-DN80 signal / yellow wire | Resistor voltage divider, then Feather D11 | D11 is the flow pulse input used by the main program. |
| Feather 3V | DS3231 VCC | Powers the RTC at 3.3 V. |
| Feather GND | DS3231 GND | Common ground. |
| Feather SDA | DS3231 SDA | I2C data connection. |
| Feather SCL | DS3231 SCL | I2C clock connection. |
| microSD card | Feather built-in SD slot | No extra SD wiring is required. The program uses CS pin D4. |

Important: the YF-DN80 requires at least about 3.5 V to operate. If the battery voltage becomes too low, the flowmeter may stop working before the Feather turns off.

# 3. Main Program Settings

The following values are near the top of FlowMeterMain.ino and can be changed before uploading:

| Setting | Current value | Purpose |
| --- | --- | --- |
| FLOW_PIN | 11 | Flowmeter signal input. Keep this at 11 unless the signal wire is moved. |
| SD_CS_PIN | 4 | Chip-select pin for the Feather's built-in SD card. |
| K_FACTOR_HZ_PER_L_MIN | 0.45 | Main flow conversion value. Change this only when a new calibration value is available. |
| CALIBRATION_SCALE | 1.0 | Optional multiplier for the calculated flow. Normally keep at 1.0. |
| FLOW_OFFSET_L_MIN | 0.0 | Optional flow offset. Normally keep at 0.0. |
| MIN_VALID_FLOW_L_MIN | 0.0 | Values below this limit are recorded as zero. It can be increased slightly if there is low-level noise when no water is flowing. |
| SAMPLE_INTERVAL_MS | 10000 | Logging interval in milliseconds. 10000 means one row every 10 seconds. |
| FORCE_SET_RTC_TO_COMPILE_TIME | false | Set to true for one upload to set the RTC, then return it to false and upload again. |

# 4. Uploading the Main Program

Use the Arduino IDE with the Adafruit SAMD board package and the RTClib library installed.

Connect the Feather M0 to the computer with a USB cable.

Open Code/FlowMeterMain/FlowMeterMain.ino in the Arduino IDE.

Select Adafruit Feather M0 as the board and select the correct COM port.

Check the program settings listed above.

Click Upload.

Open Serial Monitor at 115200 baud.

Confirm that the RTC is detected, the SD card initializes, and the message 'Logger ready' appears.

# 5. Setting the RTC

Change FORCE_SET_RTC_TO_COMPILE_TIME to true.

Compile and upload the program once.

Check the RTC time in Serial Monitor.

Change FORCE_SET_RTC_TO_COMPILE_TIME back to false and upload again.

# 6. Logging and Reading the SD Card

Insert a FAT16- or FAT32-formatted microSD card into the Feather before turning the system on. Each time the Feather starts, the program creates a new CSV file using the RTC date and time.

File path: /YYYYMMDD/HHMMSS.CSV

| CSV column | Meaning |
| --- | --- |
| timestamp | Date and time from the RTC. |
| session_elapsed_seconds | Seconds since the Feather restarted. |
| sample_number | Sample number for the current session. |
| pulses | Raw flowmeter pulse count for the sample interval. |
| frequency_hz | Pulse frequency. |
| flow_rate_l_min | Calculated flow rate in liters per minute. |
| total_volume_l | Accumulated volume since the last restart. |

## To read the data

Turn off power to the Feather.

Remove the microSD card and insert it into a computer.

Open the folder named with the logging date.

Open the CSV file in Excel, Google Sheets, or a text editor.

Note: total_volume_l resets to zero whenever the Feather restarts. It is the total for one power-on session, not a permanent lifetime total.

# 7. Quick Troubleshooting

| Problem | Check |
| --- | --- |
| No flow reading | Check the BAT power connection, common ground, voltage-divider signal connection, and D11. |
| RTC not found | Check 3V, GND, SDA, and SCL. |
| Wrong time | Use the one-time RTC setting procedure, then return the setting to false. |
| SD initialization failed | Check that the card is inserted and formatted as FAT16 or FAT32. |
| Flow value is too high or low | Check K_FACTOR_HZ_PER_L_MIN and confirm the correct calibration value. |